SDN packet routing logic with nodes and AUVs

First packet sent and finds it’s path via greedy hop (least transmission delay path chosen) , this is a multi-hop routing till the packet reaches from sender node (multiple sender nodes that shift randomly) to the destination node(node on the surface).

The stable path (path traversed by the first packet) is stored in OC flowtable (N-1 => N-5 => N-7 => AUV-3 => destination) , OC forwards the stable path via available LC(optimal LC placement chosen via SVM) , this can be multi-LC-hop till it reaches the sender node.

The successive nodes then follow the stable path stored in the flow table, reducing packet drop and delay.

If the first packet cannot find path via node to node , then fallback to AUV , If still not found then packet dropped , and it continues for rest of the packets.

Nodes at greater depth can now also be used for packet routing

How E2ED computed ?

From node to node , Total Delay computed for one hop (via the paper’s equation tranmission + propagation ) , includes idle energy of the nodes(energy whilst nodes not comuunicating). E2ED is averaged value.

For node to auv are we calculating the delay via the same paper equation?

Delay in relaying stable path from OC to sender node.

PDR :

packet considered delivered

- Sender node to destination node

- Auxiliary node to only LC (OC and AUV not involved).

ECR :

Ecomm(Edata + Ectrl) / Ecomm + Eflight(constant) + Eidle(time accumulating)

RoR :

Ctrl_bits / ctrl_bits + data_bits

Routing Mechanism

OSAR : Baseline results (with AUVs nodes)

SDN :  Flow table (stable path )

Trial and Error

Issues  increase pdr with nodes

- Dense netowrk , number of hops increase , success_probality compound effect

e.g (sparse network , 1-2 hop , 90% p_success , 0.9x0.9 = 0.81)

(dense network 5-6 hop , 0.9x0.9x0.9x0.9x0.9 = 0.53 )

Path fragility , failure of packets

- Dense  , neighbors talking simultaneously increase , p_busy increase ?

- Node drift effect

Issues increase pdr for baseline

- LC/OC selection not included in routing , treats AUVs as destination node	or Relay.

- No carrier sensing Multiple access , doesnot wait and retry for channel busy.

Solutions increase pdr with nodes

- Cost of Djisktra , Cost = Distance  => cost = 1 / (Link Reliability) , prefer shorter hops , logic = long hops higher signal failure chances.

- Node drift , local intelligence , packet node drift , temporarily shift to greedy update MC.

TRIAL 1

Solutions for baseline pdr very low

- Spatial Diversity , one neighbor busy then communicate via other nearby neighbor

Score = forward_progress * link_reliabilty

- CSMA with backoff until channel free.

Results :

PDR for 8 is 0.9 something  and for rest is 1.0 using SDN(ML).

PDR increase but at 8 nodes it should not be this high ?

PDR for baseline is very low [0.02,0.03]

Trial : Fail

Trial 2

Issuses

pdr 0.9 at 8 nodes and 1 at rest

- Once SDN path found , channel reserved , skips P_BUSY.

- Allow deeper nodes can be chosen

Baseline pdr low

- Strict upward progress rule.

Results :

PDR at 8 around 0.7 , but for >8 nodes , pdr 1.0 ?

ML/SDN RoR [0.004, 0.0] very low.

Baseline pdr [0.02 , 0.06 ] still very low.

Baseline RoR [0.95,0.98] very high

Trial : Fail

TRIAL 3

- SDN path found , channel reserved , P_BUSY ignored , but this is resolved .

- Delay/Cost/packet expiration of path repair after break not consdered so 1.0 pdr.

- RoR 0.0 , Maintenance overhead , constant “Hello messages” sent to MC.

- Control bits only considered during discovery but not during the the actual propagation via flow table path. (necessary to check if channel busy or not ).

- Strict shallower constraints should be kept, greater depth nodes rejected.

Results

PDR 8 nodes 0.78 , >8 1.0 for SDN.

RoR 0.9.

Baseline again very low pdr 0.04.

Trial : Fail

Trial 4

- Packet expiration shortened (1s).

- For SDN , when stable path found , ignore P_BUSY , CSMA add , CSMA remove.

- RoR flooding , Remove constant hello message by nodes per second.

- Baseline PDR, reliability score (P^4) , favouring short multi hops.

Results :

Trial i: SDN PDR correct(increase with increasing nodes) except SVM pdr decrease with increasing nodes.

Trial ii : Baseline PDR still very low 0.04.

Trial : Fail

Trial 5

- Priority Access system , manipulate P_BUSY to allow certain nodes to pass even if busy.

- Allower deeper nodes.

Results :

SDN PDR correct for all , rest params also correct .

Baseline PDR still very low ?

Trial : Partial Fail

Trial 6

- Exhuastive Fallback:

Phase 1 : packet sending to all good nodes (shallower depth)

Phase 2 : if Phase 1 fail , search using Bad neighbors (deeper nodes).

- Reliability Penalty reduce , P^4 to P^2.

Results :

Baseline pdr low .

Trial : Fail

Trial 7 :

- Add retry factor for Baseline only.

Fail

Trial 8 :

- Retry loop but for physics check(MAC layer ARQ) , ACK from node and retransmission to same node.

- P^4

- Hop limit set to 50.

- P_busy * 0.1

Trial : Fail

Trial 9

- Packet expiration low (1s).

- AUV prioritization , give AUV massive score.

- Nodes near the surface are very low due to the random distribution of nodes.

Results : Fail

Trial 10 :

- 20% nodes placed near shallow area.

Result :

Pdr for baseline increased to 0.9

But the pdr for SDN for nodes > 8 , converges to 1.0

Pdr for baseline should be lesser than for SDN

Trial 11

- Change P^4 to P^2.

- Reduce shallower nodes from 20 to 10 % .

- SDN contention factor , from 0.1 to 0.2

Results :

Pdr for SDN very high even for low nodes

Baseline PDr also very high ~0.9

Trial 12

- SDN contention , from 0.2 to 0.3

- Retries from 7 t o3

- Remove panic mode

- Hop limit from 50 to 20

All functions

Class Node

- Update_position

- Update_channels

Class AUV:

- Collect_Data

- Relay_data

- Update_channels

- Update

Class HeadlessSimulator

- _run_monte_carlo_point

- Run_full_sweep

- Place_nodes_randomly

- Find_greedy_next_hop

- Calculate_global_optimal_path

- Worker_init

- Worker_simulation_task

- Build_topology_graph

- Dijisktra_path

- Compute_all_paths_proactively

- Invalidate_stale_path